Deep Learning vs. Statistical Models in Energy Production Forecasting: A Comprehensive Literature Review

# Introduction

Accurate forecasting of energy production is essential for the integration and management of renewable energy sources, such as solar, wind, and hydropower, into modern power grids. The literature reveals a clear trend: deep learning (DL) models and advanced machine learning (ML) techniques consistently outperform traditional statistical models in most energy forecasting scenarios, especially when dealing with complex, nonlinear, and high-dimensional data (Sedai et al., 2023; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019). While statistical models like ARIMA and SARIMA remain useful for simpler, linear, or short-term forecasting tasks, their limitations become apparent as the complexity and variability of energy systems increase (Sedai et al., 2023; Haider et al., 2022; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019). Deep learning models—such as LSTM, GRU, CNN, and hybrid architectures—excel in capturing temporal and spatial dependencies, handling large datasets, and adapting to non-stationary patterns, leading to significant improvements in forecast accuracy and reliability (Sedai et al., 2023; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019; Luo et al., 2021; Husein et al., 2024; Mirza et al., 2023). However, challenges remain regarding data requirements, computational costs, and interpretability. This review synthesizes findings from recent comparative studies, meta-analyses, and case studies to provide a nuanced understanding of the strengths, weaknesses, and future directions for both statistical and deep learning models in energy production forecasting.

Figure 1. Consensus Meter: Do deep learning models outperform statistical models in forecasting energy production?

# Results

## Model Performance: Statistical vs. Deep Learning

Multiple studies demonstrate that deep learning models (LSTM, GRU, CNN, hybrid models) consistently outperform traditional statistical models (ARIMA, SARIMA, Prophet) in forecasting energy production, particularly for complex, nonlinear, and multivariate time series data (Sedai et al., 2023; Cabello-L´opez et al., 2023; Alkandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019; Luo et al., 2021; Husein et al., 2024; Mirza et al., 2023). For example, deep learning models reduced mean absolute error by up to 47% compared to official statistical forecasts in national-level solar energy prediction (Cabello-L´opez et al., 2023), and hybrid deep learning models achieved superior accuracy and stability in both short- and long-term forecasts (Li et al., 2020; Jamil et al., 2023; Venkateswaran & Cho, 2024).

## Application Domains and Case Studies

- Solar Energy: Deep learning models, especially CNN-LSTM hybrids, outperform statistical models in both short- and long-term solar irradiance and power output forecasting (Sedai et al., 2023; Cabello-L´opez et al., 2023; Bassey, 2023; Li et al., 2020; Jamil et al., 2023; Kumari & Toshniwal, 2021; Luo et al., 2021; Husein et al., 2024; Venkateswaran & Cho, 2024).

- Wind Energy: CNN and LSTM models show exceptional performance in wind speed and power prediction, capturing complex patterns better than ARIMA or regression models (Abdelsattar et al., 2025; Li et al., 2023; Zafar et al., 2022; Alc´antara et al., 2022; Hu et al., 2020; Mirza et al., 2023).

- Hydropower and Load Forecasting: Deep learning models, including hybrid and probabilistic approaches, provide more accurate and reliable forecasts than statistical methods, especially for multivariate and probabilistic scenarios (Ehtearm et al., 2023; Mashlakov et al., 2021; Miroshnyk et al., 2021; Fan et al., 2024; Ramos et al., 2023).

## Hybrid and Ensemble Approaches

Hybrid models that combine statistical and deep learning methods (e.g., SARIMA-Transformer, CNNLSTM, SSA-CNN-LSTM) further enhance forecasting accuracy, leveraging the strengths of both approaches (AlKandari & Ahmad, 2020; Li et al., 2020; Husein et al., 2024; Venkateswaran & Cho, 2024; Mirza et al., 2023; Padilha et al., 2022). Ensemble methods and attention mechanisms also contribute to improved performance, particularly in handling uncertainty and non-stationarity (AlKandari & Ahmad, 2020; Garine & Sharma, 2025; Ofose et al., 2025; Mirza et al., 2023; Padilha et al., 2022).

## Limitations and Challenges

- Statistical Models: Struggle with nonlinearity, high-dimensionality, and long-term forecasting; performance degrades rapidly with increased forecast horizon or data complexity (Sedai et al., 2023; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019).

- Deep Learning Models: Require large, high-quality datasets and significant computational resources; risk of overfitting and challenges in interpretability and hyperparameter tuning (Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Kumari & Toshniwal, 2021; Luo et al., 2021; Husein et al., 2024; Alazemi et al., 2024).

| Type | Name |
| --- | --- |
| Author | Huazhi Wang (Wang et al., 2019) |
| Author | Ghadah Alkhayat (Alkhayat & Mehmood, 2021) |
| Author | Xing Luo (Luo et al., 2021) |
| Journal | Applied Energy (Cabello-L´opez et al., 2023; Mashlakov et al., 2021; Jamil et al., 2023; Luo et al., 2021; Dum |
| Journal | Energies (Cantillo-Luna et al., 2023; Benrabia & S¨offker, 2025; Khortsriwong et al., 2023) |
| Journal | Renewable Energy (Haider et al., 2022; Li et al., 2023; Hu et al., 2020; Mirza et al., 2023) |

## Evaluation Metrics and Benchmarks

Common metrics include MAE, RMSE, MAPE, and R². Deep learning models consistently achieve lower error rates and higher R² values across diverse datasets and forecasting horizons (Sedai et al., 2023; Cabello-L´opez et al., 2023; Li et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021;

Li et al., 2020; Jamil et al., 2023; Kumari & Toshniwal, 2021; Luo et al., 2021; Husein et al., 2024; Venkateswaran & Cho, 2024; Mirza et al., 2023; Khortsriwong et al., 2023; Maleki et al., 2024).

Table 1: Authors & journals that appeared most frequently in the included papers.

# Discussion

The literature strongly supports the superiority of deep learning models over traditional statistical models in most energy production forecasting scenarios, especially as data complexity and forecasting horizons increase (Sedai et al., 2023; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019; Luo et al., 2021; Husein et al., 2024; Mirza et al., 2023). Deep learning models excel at capturing nonlinearities, temporal dependencies, and high-dimensional relationships, making them particularly well-suited for renewable energy forecasting where weather and environmental factors play a significant role (Sedai et al., 2023; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019; Luo et al., 2021; Husein et al., 2024; Mirza et al., 2023). Hybrid and ensemble approaches further enhance performance by combining the strengths of both model types (AlKandari & Ahmad, 2020; Li et al., 2020; Husein et al., 2024; Venkateswaran & Cho, 2024; Mirza et al., 2023; Padilha et al., 2022).

However, deep learning models are not without challenges. They require substantial data and computational resources, and their ”black box” nature can hinder interpretability and trust in operational settings (Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Kumari & Toshniwal, 2021; Luo et al., 2021; Husein et al., 2024; Alazemi et al., 2024). Statistical models remain valuable for simpler, linear, or data-scarce scenarios, and their transparency and ease of implementation are advantageous in certain contexts (Sedai et al., 2023; Haider et al., 2022; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019).

# Conclusion

Recent advances in deep learning have revolutionized energy production forecasting, with DL models consistently outperforming traditional statistical approaches in accuracy, robustness, and adaptability to complex, nonlinear, and high-dimensional data. Hybrid and ensemble models offer further improvements, while statistical models retain value in specific, simpler contexts. Ongoing challenges include data requirements, computational costs, and interpretability.

## Research Gaps

Despite significant progress, several gaps remain:

- Limited research on interpretability and explainability of deep learning models in operational energy forecasting.

- Insufficient studies on the performance of models in data-scarce or highly volatile environments.

- Few large-scale, standardized benchmarks for cross-comparison of models across diverse geographies and energy types.

# References

.5in1 Sedai, A., Dhakal, R., Gautam, S., Dhamala, A., Bilbao, A., Wang, Q., Wigington, A., & Pol, S. (2023). Performance Analysis of Statistical, Machine Learning and Deep Learning Models in Long-Term Forecasting of Solar Power Production. Forecasting. https://doi.org/10.3390/forecast5010014

Haider, S., Sajid, M., Sajid, H., Uddin, E., & Ayaz, Y. (2022). Deep learning and statistical methods for short- and long-term solar irradiance forecasting for Islamabad. Renewable Energy. https://doi.org/10.1016/j.renene.20 Abdelsattar, M., Ismeil, M., Menoufi, K., Abdelmoety, A., & Emad-Eldeen, A. (2025). Evaluating Machine Learning and Deep Learning models for predicting Wind Turbine power output from environmental factors. PLOS ONE, 20. https://doi.org/10.1371/journal.pone.0317619

Cabello-L´opez, T., Carranza-Garc´ıa, M., Riquelme, J., & Garc´ıa-Gutierrez, J. (2023). Forecasting solar energy production in Spain: A comparison of univariate and multivariate models at the national level. Applied Energy. https://doi.org/10.1016/j.apenergy.2023.121645

Li, X., Li, K., Shen, S., & Tian, Y. (2023). Exploring Time Series Models for Wind Speed Forecasting: A Comparative Analysis. Energies. https://doi.org/10.3390/en16237785

Bassey, K. (2023). SOLAR ENERGY FORECASTING WITH DEEP LEARNING TECHNIQUE.

Engineering Science & Technology Journal. https://doi.org/10.51594/estj.v4i2.1286

AlKandari, M., & Ahmad, I. (2020). Solar power generation forecasting using ensemble approach based on deep learning and statistical methods. Applied Computing and Informatics. https://doi.org/10.1016/j.aci.2019.11 Garine, R., & Sharma, S. (2025). Probabilistic Deep Learning for Energy Time Series Forecasting: A Comparative Study. 2025 13th International Symposium on Digital Forensics and Security (ISDFS), 1-6. https://doi.org/10.1109/isdfs65363.2025.11011947

Devaraj, J., Elavarasan, R., Shafiullah, G., Jamal, T., & Khan, I. (2021). A holistic review on energy forecasting using big data and deep learning models. International Journal of Energy Research, 45, 13489 - 13530. https://doi.org/10.1002/er.6679

Cantillo-Luna, S., Moreno-Chuquen, R., Celeita, D., & Anders, G. (2023). Deep and Machine Learning Models to Forecast Photovoltaic Power Generation. Energies. https://doi.org/10.3390/en16104097 Zafar, M., Khan, N., Mansoor, M., Mirza, A., Moosavi, S., & Sanfilippo, F. (2022). Adaptive MLbased technique for renewable energy system power forecasting in hybrid PV-Wind farms power conversion systems. Energy Conversion and Management. https://doi.org/10.1016/j.enconman.2022.115564

Ehtearm, M., Zadeh, H., Seifi, A., Fayazi, A., & Dehghani, M. (2023). Predicting Hydropower Production Using Deep Learning CNN-ANN Hybridized with Gaussian Process Regression and Salp Algorithm. Water Resources Management, 37, 3671-3697. https://doi.org/10.1007/s11269-023-03521-0

Li, P., Zhou, K., Lu, X., & Yang, S. (2020). A hybrid deep learning model for short-term PV power forecasting. Applied Energy. https://doi.org/10.1016/j.apenergy.2019.114216

Mashlakov, A., Kuronen, T., Lensu, L., Kaarna, A., & Honkapuro, S. (2021). Assessing the performance of deep learning models for multivariate probabilistic energy forecasting. Applied Energy. https://doi.org/10.1016/j.apenergy.2020.116405

Alkhayat, G., & Mehmood, R. (2021). A review and taxonomy of wind and solar energy forecasting methods based on deep learning. Energy and AI. https://doi.org/10.1016/j.egyai.2021.100060

Miroshnyk, V., Shymaniuk, P., & Sychova, V. (2021). Short Term Renewable Energy Forecasting with Deep Learning Neural Networks. Power Systems Research and Operation. https://doi.org/10.1007/9783-030-82926-1 6

Alc´antara, A., Galv´an, I., & Aler, R. (2022). Deep neural networks for the quantile estimation of regional renewable energy production. Applied Intelligence, 53, 8318-8353. https://doi.org/10.1007/s10489022-03958-7

Jamil, I., Lucheng, H., Iqbal, S., Aurangzalb, M., Jamil, R., Kotb, H., Alkuhayli, A., & AboRas, K. (2023). Predictive evaluation of solar energy variables for a large-scale solar power plant based on triple deep learning forecast models. Alexandria Engineering Journal. https://doi.org/10.1016/j.aej.2023.06.023 Ofose, B., Okpura, N., & Udofia, K. (2025). Comparative Analysis of Hybrid Machine Learning Models for Photovoltaic Energy Output Prediction: A Case Study from Nigeria’s South-South Region. Journal of Engineering Research and Reports. https://doi.org/10.9734/jerr/2025/v27141481

Kumari, P., & Toshniwal, D. (2021). Deep learning models for solar irradiance forecasting: A comprehensive review. Journal of Cleaner Production, 318, 128566. https://doi.org/10.1016/j.jclepro.2021.128566

Wang, H., Lei, Z., Zhang, X., Zhou, B., & Peng, J. (2019). A review of deep learning for renewable energy forecasting. Energy Conversion and Management. https://doi.org/10.1016/j.enconman.2019.11799

Luo, X., Zhang, D., & Zhu, X. (2021). Deep learning based forecasting of photovoltaic power generation by incorporating domain knowledge. Energy, 225, 120240. https://doi.org/10.1016/j.energy.2021.120240 Benrabia, I., & S¨offker, D. (2025). Modeling and Evaluation of Forecasting Models for Energy Production in Wind and Photovoltaic Systems. Energies. https://doi.org/10.3390/en18030625

Husein, M., Gago, E., Hasan, B., & Pegalajar, M. (2024). Towards energy efficiency: A comprehensive review of deep learning-based photovoltaic power forecasting strategies. Heliyon, 10. https://doi.org/10.1016/j.heliyon.202

Dumas, J., Wehenkel, A., Lanaspeze, D., Cornelusse, B., & Sutera, A. (2021). A deep generative model for probabilistic energy forecasting in power systems: normalizing flows. Applied Energy. https://doi.org/10.1016/j.apenergy.2021.117871

Hu, H., Wang, L., & Lv, S. (2020). Forecasting energy consumption and wind power generation using deep echo state network. Renewable Energy, 154, 598-613. https://doi.org/10.1016/j.renene.2020.03.042

Venkateswaran, D., & Cho, Y. (2024). Efficient solar power generation forecasting for greenhouses: A hybrid deep learning approach. Alexandria Engineering Journal. https://doi.org/10.1016/j.aej.2024.02.004

Alazemi, T., Darwish, M., & Radi, M. (2024). Renewable energy sources integration via machine learning modelling: A systematic literature review. Heliyon, 10. https://doi.org/10.1016/j.heliyon.2024.e26088

Mirza, A., Shu, Z., Usman, M., Mansoor, M., & Ling, Q. (2023). Quantile-transformed multiattention residual framework (QT-MARF) for medium-term PV and wind power prediction. Renewable Energy. https://doi.org/10.1016/j.renene.2023.119604

Fan, G., Han, Y., Li, J., Peng, L., Yeh, Y., & Hong, W. (2024). A hybrid model for deep learning short-term power load forecasting based on feature extraction statistics techniques. Expert Syst. Appl., 238, 122012. https://doi.org/10.1016/j.eswa.2023.122012

Padilha, G., Ko, J., Jung, J., & De Mattos Neto, P. (2022). Transformer-Based Hybrid Forecasting Model for Multivariate Renewable Energy. Applied Sciences. https://doi.org/10.3390/app122110985

Khortsriwong, N., Boonraksa, P., Boonraksa, T., Fangsuwanarak, T., Boonsrirat, A., Pinthurat, W., & Marungsri, B. (2023). Performance of Deep Learning Techniques for Forecasting PV Power Generation:

A Case Study on a 1.5 MWp Floating PV Power Plant. Energies. https://doi.org/10.3390/en16052119

Ramos, P., Villela, S., Silva, W., & Dias, B. (2023). Residential energy consumption forecasting using deep learning models. Applied Energy. https://doi.org/10.1016/j.apenergy.2023.121705

Maleki, N., Lundstr¨om, O., Musaddiq, A., Jeansson, J., Olsson, T., & Ahlgren, F. (2024). Future energy insights: Time-series and deep learning models for city load forecasting. Applied Energy.

https://doi.org/10.1016/j.apenergy.2024.124067

|  | Solar Forecasting | Wind Forecasting | Hydropower Forecasting | Load Foreca |
| --- | --- | --- | --- | --- |
|  | (# papers) | (# papers) | (# papers) | (# papers |
| Statistical Models | 12 | 8 | 2 | 7 |
| Deep Learning Models | 18 | 13 | 3 | 10 |
| Probabilistic Forecasts | 5 | 4 | 1 | 2 |

| Claim | Evidence Strength | Reasoning | Papers |
| --- | --- | --- | --- |
| Deep learning models outperform statistical models in complex energy forecasting tasks | Strong | Multiple comparative studies and meta-analyses show lower error rates and higher accuracy for DL models | (Sedai et al., 2023; Cabello-L´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019; Luo et al., 2021; Husein et al., 2024; Mirza et al., 2023) |

Table 2: Key claims and support evidence identified in these papers.

| Hybrid models (statistical + DL) further improve forecasting accuracy |  | Strong | Hybrid and ensemble approaches leverage strengths of both model types, reducing errors and improving robustness | (AlKandari & Ahmad, 2020; Li et al., 2020; Husein et al., 2024; Venkateswaran & Cho, 2024; Mirza et al., 2023; Padilha et al., 2022) |  |
| --- | --- | --- | --- | --- | --- |
| Statistical models are suitable for simple, linear, or short-term forecasts | Moderate | Statistical models perform well in low-complexity, short-horizon, or datascarce scenarios | (Sedai et al., 2023; Haider et al., 2022; CabelloL´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019) |
| Deep learning models require large datasets and high computational re- sources |  | Moderate | DL models’ performance depends on data volume and quality; computational costs can be significant | (Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Kumari & Toshniwal, 2021; Luo et al., 2021; Husein et al., 2024; Alazemi et al., 2024) |  |
| Interpretability and transparency are greater in statistical models | Moderate | Statistical models are more transparent and easier to interpret, aiding operational trust | (Sedai et al., 2023; Haider et al., 2022; CabelloL´opez et al., 2023; AlKandari & Ahmad, 2020; Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Wang et al., 2019) |
| Deep learning models risk overfitting and require careful tuning |  | Moderate | Overfitting and hyperparameter sensitivity are common challenges in DL model deployment | (Devaraj et al., 2021; Alkhayat & Mehmood, 2021; Kumari & Toshniwal, 2021; Luo et al., 2021; Husein et al., 2024; Alazemi et al., 2024) |  |

Table 3: Heatmap of research coverage by topic and model type; gaps indicate underexplored areas.